Documentation - Supply Chain Game

- Connecting Raspberry Pi to Wi-Fi: Bayern@WLAN

- Browser

hotspot.vodafone.de/Bayern

- If it does not work:

  - Open terminal: sudo date --set 'JJJJ-MM-TT HH:MM:SS‘ (type in the current date and time)

  - Try connection again

- Updating Raspberry Pi

  - Terminal

sudo apt-get update

sudo apt-get upgrade

- Download Git-Repository (if it not exists on the desktop)

  - Terminal: cd Desktop (or Schreibtisch)

  - Terminal: git clone https://github.com/IoTLabHNU/SCGame

- Installing RFID-Modul

  - GPIO Assignment:

| RFID-Reader | Raspberry Pi |
| --- | --- |
| VCC | GPIO 1 (3V Power) |
| RST | PIN 22 (BCM 25) |
| GND | PIN 6 (Ground) |
| MISO | PIN 21 (BCM 9) |
| MOSI | PIN 19 (BCM 10) |
| SCK | PIN 23 (BCM 11) |
| NSS | PIN 24 (BCM 8) |
| IRC | / |

- Circuit diagram

- SPI

  - Activate SPI:

sudo raspi-config

interfacing options – SPI – enable

sudo reboot

  - Updating Config-file

sudo nano /boot/config.txt

dtparam=spi=on (add at the end of the file)

  - Installing RFID-Reader

sudo apt-get install python3-dev python3-pip sudo pip3 install spidev

sudo pip3 install mfrc522

  - Test RFID-Reader:

Open: Read.py or Write.py

- Installing MySQL-Server on rp-admin

- Terminal

sudo apt-get install mysql-server

sudo mysql -u root -p

enter password: root

create user ‚pi‘@‚%‘ identified by ‚pi’;

grant all privileges on *.* to ‚pi‘@‚%‘ with grant option;

flush privileges;

exit

- Bind-Adress

  - Terminal:

sudo nano /etc/mysql/mariadb.conf.d/50-server.cnf

Change Bind-adress to 0.0.0.0

strg + o

strg + x

- Change name of all Raspberry PIs

- Terminal:

sudo nano /etc/hostname

change name to:

rp-admin

rp-team1

rp-team2

rp-team3

rp-team4

strg + o, enter

strg + x

- Terminal

sudo nano /etc/hosts

change name to:

rp-admin

rp-team1

rp-team2

rp-team3

rp-team4

stag + o

strg + x

- Reboot

sudo reboot

- Installing MySQL Database Driver

- Terminal

sudo apt-get install python-mysqldb

sudo python -m pip install mysql-connector